H_
MATERIAL SAFETY DATA SHEET	F

SECTION I - PRODUCT IDENTIFICATION

TRADE NAME:  CONCRETE MIX  - SAKRETE, STTRE_MIX, 3ONFAL CHEMICAL FAMILY/SYNONYMS:  Portland Cement Mortar, lime, silica MANUFACTURER:  W. R. BONSAL COMPANY, P.O. BOX 241148, CHARLOTTE, N. C. 28224 EMERGENCY PHONE:  (704) 525-1621     Mr. Fred Goeman

SECTION II - HAZARDOUS INGREDIENTS

DESCRIPTION	PERCENT     OCCUPATIONAL         VAPOR

EXPOSURE LIMITS      PRESSURE

CAS #	   TLV	PEL	(MM HG)

HYDRAULIC CEMENT	65997-15-1    15-18	10 mg/m3	N/A

QUARTZ*	14808-60-7    12	BELOW	N/A

CRUSHED STONE (ROCK

GRAVEL)	1317-65-3	30-40	10 mg/m3

CRYSTOBALITE	14464-46-1	BELOW

SAND (FIELD SAND)	7631-86-9	30-40

AMORPHOUS SILICA	60676-86-0	10-20	10 mg/m3

10 mg/m2

TLV (QUARTZ) =	  = PEL (QUARTZ)

% respirable quartz + 2

NOTE: Previous air monitoring during employee exposure to this product has indicated typical airborne respirable quartz fractions of up to 3%. TLV: Threshold Limit Value established by the American Conference of Governmental Industrial Hygienists, 1985-86.

SECTION III - PHYSICAL DATA

BOILING POINT (°F)       N/A         SPECIFIC GRAVITY (H20=l)	2.6 2.7

VAPOR PRESSURE (mmHg.)   N/A         % VOLATILE BY VOLUME (*)	Q

VAPOR DENSITY (AIR=1)    N/A         EVAPORATION RATE	N/A

SOLUBILITY IN WATER:  Slight	(0.1-1.0%)

APPEARANCE & ODOR:  Fine, grey powder mixed with sand and/or rock, no odor.

SECTION IV - FIRE & EXPLOSION HAZARD DATA

FLASH POINT:   Will not ignite	   FLAMMABLE LIMITS      N/A

EXTINGUISHING MEDIA:  None

SPECIAL FIRE FIGHTING PROCEDURES:  None

UNUSUAL FIRE AND EXPLOSION HAZARD :  None

100	Page 1 of 2	JAN-90

SECTION V - HEALTH HAZARD DATA THRESHOLD LIMIT VALUE:  Respirable Dust  - TLV 10MG/M3

EFFECTS OF OVEREXPOSURE:  Excess exposure by inhalation over an extended period of time may result in the development of pulmonary diseases including pneumoconiosis and silicosis.  Prolonged contact can produce skin irritation and dermatitis.  Abrasive, skin and eye irritant.

EMERGENCY FIRST AID: Inhalation: remove from exposure. If not breathing, give artificial respiration, if breathing difficult, give oxygen, consult a physician.  Eye Contact: flush thoroughly with water for 15 minutes. Consult a physician.  Skin Contact: wash thoroughly with soap and water.

SECTION VI - REACTIVITY DATA

STABILITY:	UNSTABLE 	      STABLE   X

CONDITIONS TO AVOID: N/A

INCOMPATABILITY (MATERIALS TO AVOID): Highly alkaline. Strong acids may cause violent, exothermic reaction, evolve toxic gases/vapors, depending on type acid. HAZARDOUS DECOMPOSITION PRODUCTS:  N/A

HAZARDOUS POLYMERIZATION:	MAY OCCUR 	    WILL NOT OCCUR _X_

CONDITIONS TO AVOID:  N/A

SECTION VII - SPILL OR LEAK PROCEDURES

IN CASE MATERIAL IS RELEASED OR SPILLED:  Use personal protective equipment

to prevent dust exposures and protect against alkali burns or irritation. Sec

8.

WASTE DISPOSAL METHOD:  Dispose of in accordance with Federal, State and

local regulations.

SECTION VIII - SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION (TYPE):  NIOSH/MSHA approved for protection against

silica and nuisance dusts.

VENTILATION:

Local Exhaust: Yes	Special:

Mechanical:	Other:

PROTECTIVE GLOVES: Rubber, PVC, Neoprene   EYE PROTECTION: Goggles

OTHER PROTECTIVE EQUIPMENT: Rubber boots, arm sleeves and aprons when necessar:

to prevent skin contact.

SECTION IX - SPECIAL PRECAUTIONS

PRECAUTIONS IN HANDLING AND STORING: Store in manner to prevent contact with strong acids.  Avoid contact with water to preserve product. OTHER PRECAUTIONS:  None

100	Page 2 of 2

Prepared By: